Using the Gel Documentation System to Image (in SC basement)

DNA Agarose Gel with Ethidium Bromide as the Stain

1. Turn on the switch for the tan power strip on the left-hand side of the illumination hood (purple box)

2. Log into the desktop computer (need UD student name and password)

3. Open Vision Works software using the shortcut on the computer desktop

4. Put gel on tray into center of the hood and on top of the light box

5. Software settings for Acquisition:

Select Filter as “Eth. Brom.”

Transillumination: select UV to turn on light and select OFF later when done

Press “PREVIEW” to see gel

Can zoom in or out by adjusting the tray height

Can brighten or darken image by adjusting the aperture

Can also focus if the image is blurry

** If you want to purify DNA later, it is best to minimize time of exposure to UV light. **

6. Press “CAPTURE” to take picture of gel

7. Print picture to the Mitsubishi printer with portrait setting and 1280x1280

8. When done. Remove gel from hood. Turn off computer. Turn off the power strip. Take your picture

9. Spray some ethanol on the light box and wipe off with a kim wipe

Last Updated on 01/16/2023 by T. Williams